Nemo Project

NemoSim Simulator Tool

User Guide

Rev. 1.3

November 2025

Documentation Control

History Table

| Version | Date | Description | Author/Remarks |
| --- | --- | --- | --- |
| 1.0 | 31 July 2025 | Initial release | Marika Klubakov |
| 1.1 | 7 August 2025 | Revised and expanded | Marika Klubakov |
| 1.2 | 19 August 2025 | In Section ‎2.1: Updated and explained the LIF and BIU network XML codes Added a note about the optional LIF YFlash field In Section ‎2.2, added a note about the optional BIU sup_xml_config_path field In Section ‎2.4, reformatted the filename syntax In Section ‎3, added an important note | Ibrahem Saed Ahmd |
| 1.3 | 18 November 2025 | In section 2.1 : - added optional xml codes for manipulating specific BIU neurons parameters - updated the params table 2-2 with new parameters for configuring digital to spike converter “DS” (bit width, clock, mode) in section 2.2 : - step 2 : And two new config file keys (csv files) for calculating energy (synapses_energy_table_path, neuron_energy_table_path) . | Ibrahem Saed Ahmd |

Approval Table

| Version | Date | Full Name | Title | Signature |
| --- | --- | --- | --- | --- |
| 1.0 | 31/7/25 | Marika Klubakov |  | Marika Klubakov |
| 1.1 | 7/8/25 | Marika Klubakov |  | Marika Klubakov |
| 1.2 | 19/8/25 | Marika Klubakov |  | Marika Klubakov |
| 1.3 |  |  |  |  |

Disclaimer and Proprietary Information Notice

The information contained in this document does not represent a commitment on any part by Ceva, Inc., or its subsidiaries (collectively, "Ceva"). Ceva makes no warranty of any kind with regard to this material, including, but not limited to implied warranties of merchantability and fitness for a particular purpose whether arising out of law, custom, conduct, or otherwise.

Additionally, Ceva assumes no responsibility for any errors or omissions contained herein, and assumes no liability for special, direct, indirect, or consequential damage, losses, costs, charges, claims, demands, fees, or expenses, of any nature or kind, which are incurred in connection with the furnishing, performance, or use of this material.

This document contains proprietary information, which is protected by U.S. and international copyright laws. All rights reserved. No part of this document may be reproduced, photocopied, or translated into another language without the prior written consent of Ceva. CEVA® is a registered name of Ceva. All product names are trademarks of Ceva, or of its applicable suppliers if so stated.

Support

Ceva makes great efforts to provide a user-friendly software and hardware development environment. Along with this, Ceva provides comprehensive documentation, enabling users to learn and develop applications on their own. Due to the complexities involved in the development of DSP applications that might be beyond the scope of the documentation, an online Technical Support Service has been established. This service includes useful tips and provides fast and efficient help, assisting users to quickly resolve development problems.

How to Get Technical Support:

FAQs: Visit our website http://www.ceva-ip.com or your company's protected page on the Ceva website for the latest answers to frequently asked questions.

Application Notes: Visit our website http://www.ceva-ip.com or your company's protected page on the Ceva website for the latest application notes.

Email: Use the Ceva central support email address ceva-support@ceva-ip.com. Your email will be forwarded automatically to the relevant support engineers and tools developers who will provide you with the most professional support to help you resolve any problem.

License Keys: Refer any license key requests or problems to
sdtkeys@ceva-ip.com. For SDT license keys installation information, see the SDT Installation and Licensing Scheme Guide.

Email: ceva-support@ceva-ip.com

Visit us at: www.ceva-ip.com

Table of Contents

1.	Introduction	1

1.1	Overview	1

1.2	Purpose	1

1.3	Key Features	1

1.4	Typical Use Cases	2

2.	Using the NemoSim Simulation Tool	3

2.1	Step 1: Preparing the XML/TXT Files	4

2.2	Step 2: Preparing the JSON File	10

2.3	Step 3: Running the NemoSim Simulator Tool	11

2.4	Step 4: Plotting the Outputs	12

3.	Error Handling	15

3.1	Error Message Formats	15

3.2	Possible Return Codes	15

4.	References	17

List of Examples

Example ‎2-1: NemoSim Progress Messages	11

List of Tables

Table ‎2-1: LIF Network XML Configuration Parameter Definitions	5

Table ‎2-2: BIU Network XML Configuration Parameter Definitions	8

# Introduction

## Overview

This document describes the NemoSim simulation tool, which is used for spiking neural network architectures, and has been developed as part of the Nemo Project.

It enables researchers and engineers to model, simulate, and analyze the behavior of various neural network types, such as Leaky Integrate-and-Fire (LIF) and Brain-Inspired Unit (BIU) networks.

## Purpose

NemoSim is designed to:

Simulate neural network architectures using user-defined configurations

Model time-evolving neuron and synapse states in response to input currents

Support both standard and custom network types (LIF, BIU, and extensible for new models)

Generate outputs for scientific analysis and visualization, including detailed time-series data for neural state, synapse activity, and spikes

Facilitate debugging and research, with outputs and tools to identify and analyze network behavior

## Key Features

Flexible Input: NemoSim accepts XML-based network descriptions and plain text current input files, supporting both manual and scripted generation.

Extensible Design: The modular codebase allows for easy addition of new neuron/synapse models and architectures.

Comprehensive Output: The tool produces multiple output files (membrane potentials, synaptic inputs, spikes, and so on) for analysis and plotting.

Analysis Tools: NemoSim includes Python scripts and guidelines for visualizing and interpreting simulation results.

Robust Error Handling: The tool detects and reports errors in input files and configuration, aiding reproducibility.

## Typical Use Cases

Computational neuroscience research

Neuromorphic hardware prototyping and validation

Algorithm development and prototyping for spiking neural networks

# Using the NemoSim Simulation Tool

To use the NemoSim simulation tool, do the following:

Prepare the XML/TXT files, as described in Section ‎2.1

Prepare the JSON file, as described in Section ‎2.2

Run the NemoSim tool, as described in Section ‎2.3

Plot the output, as described in Section ‎2.4

## Step 1: Preparing the XML/TXT Files

Do the following:

- In an XML configuration file, define your network architecture, neuron parameters, and simulation settings.

| Important: | - The XML file must have a <NetworkConfig> root with a type attribute. - Required child elements depend on the network type (LIF, BIU, and so on). - All required numeric fields must be present and valid numbers. |
| --- | --- |

- For example:

For a LIF network:

<NetworkConfig type="LIF">

<LIFNetwork>

<Cm>1.0</Cm>

<Cf>0.5</Cf>

<VDD>2.5</VDD>

<!-- other LIF parameters -->

</LIFNetwork>

<Architecture>

<Layer size="32"/>

<YFlash rows="32" cols="32">

<weights>

…

</weights>

</YFlash>

<!-- network connectivity, layers, weights, etc. -->

</Architecture>

</NetworkConfig>

| Note: | YFlash is a matrix of weights, in which each weight holds a value that represents a resistance or diode that will be multiplied by the output of each neuron. Each neuron connects to the corresponding row in the YFlash, and each column of the YFlash is connected to the corresponding entrance of each neuron in the next layer. |
| --- | --- |

- Table ‎2-1 defines the key parameters that are used in this section of the LIF XML configuration file.

- These values are based on standard analog neuron circuit modeling practices and can be adapted depending on the simulation context.

Table ‎2-1: LIF Network XML Configuration Parameter Definitions

| Parameter | Description | Unit | Example Value | Notes |
| --- | --- | --- | --- | --- |
| Cm | Membrane capacitance | Farads (F) | 1e-6 | Determines the neuron integration time constant |
| Cf | Feedback capacitance | Farads (F) | 1e-9 | Affects how quickly the membrane voltage resets or leaks after a spike |
| VDD | Supply voltage | Volts (V) | 5.0 | Sets the upper voltage bounds for circuit behavior |
| VTh | Threshold voltage | Volts (V) | 1.0 | Defines the spike generation threshold |
| K | Gain factor | N/A | 2 | Dimensionless scaling factor (for example, for current mirror) |
| Rmin/Rmax | Min/Max resistance in synaptic array | Ohms (Ω) | 145e3/145e6 | Used for dynamic range in weight representation |
| gm | Transconductance | Siemens (S) | 6.896e-6 | Typically from the input differential pair |
| CGB*, CGD*, CDB* | Parasitic capacitances (Gate-*, Drain-*, Bulk-*) | Farads (F) | ~e-18 values | Derived from transistor model extraction |
| req | Effective resistance | Ohms (Ω) | 145e3 | Used in equivalent RC modeling |
| R_da | Driver array resistance | Ohms (Ω) | 10e3 | Series resistance from driver circuitry |
| dt | Simulation time step | Seconds (s) | 0.001 | Time increment for each simulation step |
| IR | Input current scaling factor | Amperes (A) | 1e-10 | Multiplies the provided input values to obtain actual currents |

For a BIU network:

<NetworkConfig type="BIU">

<BIUNetwork>

<!-- BIU parameters here -->

</BIUNetwork>

<Architecture>

<Layer size="1">

<synapses rows="1" cols="1">

<weights>

<row>7.0</row>

</weights>

</synapses>

<NeuronRange start="0" end="2"> // optional

<VTh>0.5</VTh>

<RLeak>550e6</RLeak>

<refractory>10</refractory>

</NeuronRange>

<Neuron index="6"> // optional

<VTh>0.19</VTh>

</Neuron>

</Layer>

<!-- architecture details -->

</Architecture>

</NetworkConfig>

| Note: | - The following optional repeated tags are available under each <Layer>: - <NeuronRange start="S" end="E"> with any of the following parameters: <VTh>, <refractory>, <RLeak> - <Neuron index="i"> with any of the following parameters: <VTh>, <refractory>, <RLeak> |
| --- | --- |

- Each layer has its own matrix of synapses that connects to the entrance of each neuron in the layer.

- Table ‎2-2 defines the key parameters that are used in this section of the BIU XML configuration file.

- These values are based on a switched-capacitor SNN architecture using programmable digital weights, charge sharing, and comparator-based thresholding.

Table ‎2-2: BIU Network XML Configuration Parameter Definitions

| Parameter | Description | Unit | Example Value | Notes |
| --- | --- | --- | --- | --- |
| Cu | Synapse unit capacitance | Femtofarads (fF) | 4 | Defines the granularity of weight resolution |
| Cn | Neuron integration capacitor | Femtofarads (fF) | 1000 | Large capacitor for temporal integration |
| VDD | Supply voltage | Volts (V) | 1.2 | Nominal voltage for digital and analog circuitry |
| VTh | Comparator threshold voltage | Volts (V) | ±0.4 | Spike generation threshold (programmable) |
| WS | Weight sign | N/A | +1 or -1 | Positive = VDD Negative = VSS |
| W[3:0] | Synaptic weight (4-bit) | Digital | 0–15 | Multiplies the charge contribution per synapse |
| Refractory | Refractory period (cycles) | Integer | 1–8 | Input is blocked for N cycles after spike |
| Cl | Leakage capacitance (optional) | Femtofarads (fF) | 5e-15 | Models subthreshold leakage or passive decay |
| Vm | Neuron potential | Volts (V) | Dynamic | Computed via charge-sharing at each phase |
| Nu | Number of synaptic inputs | N/A | Varies by layer | Determines the total Cu contribution per neuron |
| DSBitWidth | Digital-to-spike converter bit width | Integer | 4/8 | Determines the total Cu contribution per neuron |
| DSClockMHz | Digital-to-spike converter clock frequency | Integer | 10 | Determines the total Cu contribution per neuron |
| DSMode | Activation mode of digital-to-spike converter bit width | String | ThresholdMode ¦ FrequencyMode | Determines the activation mode of the DS |

In a plain TXT file, define the neuron values.

| Important: | Each line shall contain values corresponding to the neurons in the first layer (without delimiters), representing the input current for each time step (or input channel). |
| --- | --- |

- For example:

1 0 1 0 1 1 1 0

0 0 1 0 1 1 1 0

0 0 1 0 1 1 1 0

0 0 1 0 1 1 1 0

1 0 1 0 1 1 1 0

| Tip: | You can use the provided Python input_creator.py script to generate input files; for example: python input_creator.py > "1e-10 * math.sin(2 * math.pi * t * 5 + 3 * math.pi/2) + 1e-10" |
| --- | --- |

## Step 2: Preparing the JSON File

Do the following:

In a JSON file, define your workspace parameters and input files paths.

- For example:

For a LIF network:

{

"output_directory": "./Tests/SNN/LIF/sin_current_test/",

"xml_config_path": "./Tests/SNN/LIF/sin_current_test/testFull.xml",

"data_input_file": "./Tests/SNN/LIF/sin_current_test/input.txt",

"progress_interval_seconds": 2

}

For a BIU network:

{

"output_directory": "./Tests/SNN/BIU/output_directory",

"xml_config_path": "./Tests/SNN/BIU/test.xml",

"sup_xml_config_path": "./Tests/SNN/BIU/supervisor.xml",

"data_input_file": "./Tests/SNN/BIU/input.txt",

"synapses_energy_table_path": "./Tests/SNN/BIU/Spike-in_vs_Not_spike-in.csv",

"neuron_energy_table_path": "./Tests/SNN/BIU/Energy_Neuron_CSV_Content.csv",

"progress_interval_seconds": 2

}

| Note: | - sup_xml_config_path is an optional field used only by the BIU network. - neuron_energy_table_path is the path to the neuron energy CSV file. - synapses_energy_table_path is the path to the synapse energy CSV file. These files are read by parseConfigFromFile and copied into NetworkParameters (overriding the XML values if present). |
| --- | --- |

## Step 3: Running the NemoSim Simulator Tool

Do the following:

From the root directory, type:

NEMOSIM.exe <name of JSON file>

- For example:

NEMOSIM.exe config.json

- While the NemoSim is running, it will display progress messages on the screen, as demonstrated in Example ‎2-1. If an error occurs, an error or warning message will be displayed (for details, see Section ‎3).

Example ‎2-1: NemoSim Progress Messages

## Step 4: Plotting the Outputs

When the NemoSim has finished running, it generates output files and places them in the output directory you specified in the JSON file (as described in Section ‎2.2).

Output files are plain text, each containing a list of numeric values (one per line). Each output file corresponds to a specific neural variable, where <x> is the number of the layer, and <y> is the number of the neuron:

LIF Simulation (three files per neuron):

Iins_<x>_<y>.txt: Input currents

Vms_<x>_<y>.txt: Membrane potentials

Vouts_<x>_<y>.txt: Output voltages (spikes)

BIU Simulation (three files per neuron):

Vin_<x>_<y>.txt: Synapse input values

Vns_<x>_<y>.txt: Neural state potentials

Spikes_<x>_<y>.txt: Output spikes

After the output files have been generated, you can plot or analyze them to look for neurons with unexpected behaviors (for example, constant potentials or no spikes) for further debugging.

Do one of the following:

For LIF networks, use the plot_vm_to_dt.py script.

For BIU networks, use the plot_vn_to_dt.py script.

For example:

python plot_vm_to_dt.py Iins_0_0.txt vms_0_0.txt Vouts_0_0.txt

# Error Handling

| Important: | All errors are considered to be critical, and will stop the NemoSim Simulator. |
| --- | --- |

## Error Message Formats

Errors (examples):

Error loading XML file: <description>

Error: No <NetworkConfig> root element found.

Error: Network type attribute not found in <NetworkConfig>.

## Possible Return Codes

The code uses TinyXML2, which defines error codes such as:

XML_SUCCESS (0): Success

XML_NO_ATTRIBUTE

XML_WRONG_ATTRIBUTE_TYPE

XML_ERROR_FILE_NOT_FOUND

XML_ERROR_FILE_COULD_NOT_BE_OPENED

XML_ERROR_FILE_READ_ERROR

XML_ERROR_PARSING_ELEMENT

For the full list, see TinyXML2's XMLError enum.

# References

For details and example files for LIF, see Tests/SNN/LIF/.

For details and example files for BIU, see Tests/SNN/BIU/.

Output file examples: Vin.txt, Vns.txt (in the test folders).